| No | Requirement Specification | Proposed Specification by Vendor (please mention the detail specification) |
| --- | --- | --- |
|  |  | Vendor Name: |
| A | 1 Unit: Photocopy Machine, with minimum requirement as follows: Main Specification: - Main Function: Copy, Print, Scan, Fax (Optional) - Writing Method: Semiconductor Laser CPU ARM A53 1.6GHz - Print Speed (A4) (Mono/Color): 35/35 ppm (Minimal) - Scanning Speed (A4): 300dpi DP-7140 Simplex 50 ipm Duplex: 16 ipm - Scanning Speed (A4): 300dpi DP-7150 Simplex 80 ipm Duplex : 48 ipm - Scanning Speed (A4): 300dpi Dual Scan, DP-7160 Simplex 100 ipm, Duplex 200 ipm - Scan with Duplex Feed Detection DP: DP-7170 Simplex 100 ipm, Duplex 200 ipm - Memory: Standard 4GB (Max. 4GB) - SSD/HDD Capacity: SSD 32GB (Standard) / HDD 320GB or 1TB (Optional) - Stable Time (From the mainpower ON): 18 seconds - Paper Capacity: Standard 2 x 500 sheets (Cassette) + 150 sheets (MPT) (A4 / Letter size or less equivalent to 80 g/m2) - 2 x 600 sheets (Cassette) + 165 sheets (MPT) (A4 / Letter size or less equivalent to 64 g/m2) - Maximum: 7,150 sheets (Main Unit + 2 x 1,500 sheets PF-7150 x1 + 3,000 sheets PF-7120 x1 + MPT) - Paper Size: Cassette Min. A6R - Max. SRA3(320mm x 450mm); Cassette 1 Min. A6R – Max. A4R (Legal) - MPT Min. A6R - Max. SRA3(320mm x 450mm) & Banner Print (1,220mm x 304.8mm) - Paper Size Cassette: 52~300g/m2 - MPT 52~300g/m2 ; Banner paper 136~163g/m2 - Duplex Printing: Paper size: Min. A6R - Max. SRA3(320mm x 450mm) ; Paper weight: 60~256g/m2 - Output Capacity: Max. 500 sheets + 100 sheets face-down with Inner Job Separator - Trays: 4 trays Copy Functions (Standard): - Copy Size: Max. A3 (Ledger) - Min. A6R (StatementR) - Resolution: Scanning: 600 x 600 dpi ; Writing: 1200 x 1200 dpi multiple value - First Copy Time (Mono/Color): 6.4/8.5 seconds - Zoom Ratio: Manual - Multiple: Copy 1 - 9,999 Print Functions (Standard): - Resolution: 1,200 x 1,200 dpi ; 4,800 equivalent x 1,200 dpi level - PDL & Emulation: PRESCRIBE, PCL6 (PCL-XL, PCL 5c), KPDL3 (Postscript 3 compatible), XPS, OpenXPS, PDF Direct Print Ver. 2.0 - First Print Out Time: (Mono/Color) 7.0/9.2 seconds - Supported Protocol: TCP/IP(IPv4/IPv6), Bonjour - Operating System Supported: Windows 8.1/10, Windows Server 2012/2012 R2/2016/2019, - Mac OS X (10.9～), Google Chrome OS - Interface: Ethernet (1000Base-T/100Base-TX/10Base-T)x1, USB 3.0x1, USB Host Interface (USB Host)x4, NFC TAGx1 Scan Functions (Standard): - Original size Table: Max. A3 (Ledger) - DP: Max. A3 (Ledger) - Min. A6R (DP-7140 is A5R) - Banner: Max. 1,900mm - Resolution: 600 x 600 dpi, 400 x 400 dpi, 300 x 300 dpi, 200 x 200 dpi, 200 x 100 dpi, 200 x 400 dpi - File Format: TIFF, JPEG, XPS, OpenXPS, PDF (MMR/JPG compression/High-compression PDF), PDF/A-2, Encrypted PDF, - Searchable: PDF (Optional), MS Office file (Optional) - Image Type Mode: Text, Photo, Text + Photo, Text (Fine Line) - Send Method: SMB (Version 3.0), SMTP, support 16 length character password, FTP, FTPoverSSL, USB - TWAIN Source: (Network), WIA Driver (Network), DFS, DSM Fax Functions (Optional) |  |
| B | Maintenance: - Total FSMA Service Cost / Full-Service Maintenance Agreement (Service Contract) - Includes the supply of all consumables including Toner / Ink, Drum, Developer, Service Maintenance, including replacement of spare parts both periodically replaced and replaced due to damage (except paper). - An immediate temporary replacement with a similar capability (function) of photocopy machine should be provided by the Vendor whenever the machine is out of order for more than 1 x 24 hour. - If it is needed, vendor has to provide technical services and solution in maximum +/- 3 hours after notification from customer. - A guarantee that machine will run without any disruption (lack of tonner, circuit shortage, technical difficulties, etc) - The Selected Vendor should provide cost per page for additional copy (Colour and BW). - A fully support in providing commercial invoice, VAT invoice and supporting documents for payments and tax exemption as required by GIZ. |  |